ECE 6323 LAB Report

Submitted BY  BIJON SAHU (#1647781)

****************************************************************************************************************************************

Q1. (20 pts) Detector noise

We discussed in class that dark current is undesirable. Use the APP to illustrate why with an example (compare a case of high and low dark current).

Answer:

Dark current carries a current noise inoise, that, from the lecture, we can see:

i^2 = 2 e *idark* B

Where e is the electron charge, idark is the dark current, and B is the bandwidth. To illustrate with an example, let’s consider the condition below. We have a detector with 10 nA dark current. The simulation indicates that dark current noise is very small compared with the signal noise; hence it is not a problem.

Figure 1: Low dark current better BER

Dark Current = 10 nA  produces a BER=approx.(10 ^-5.7)

Optical Power signal :-26 db ,  RIN = -155 db ,  Wavelength = 1501.21 nm ,AMP Temperature= 300K

Detector  Responsivity : 0.5 A/W , Low Dark current = 10 nA  , Electronic Resistance = 10 K ohm, NF=3db

Sampling Rate = 10 GS/s , Samples = 5000

Figure 2: Higher dark current low  BER

Higher Dark Current = 10 uA produces a BER=approx.(10^-4)

Q2. (40 pts) Thermal (Johnson) noise

We discussed in class that even if we have a good - low-dark current detector, noisy electronics which include low equivalent-impedance TIA (trans impedance amplifier) and voltage amplifier with high NF can also degrade the BER. Use the APP to illustrate these points with examples .

Answer:

Even without dark current, any signal also has statistical fluctuation that obeys Poisson statistics:

nave  =  isignal Δt/e

P[n] = Subscript[n, ave]n/n!

The corresponding distribution of the current measurement is:

P[i] ≈ 1/( σi)

where σi = e/Δt. Notice that:

Subscript[σ, i]2 =(nave e2)/Δt2=  isignal e/Δt =isignal e 2B

Or:                                            =2 e isignal B

Here in the below use cases from 1a to 1c  we are trying to capture BER by  varying temperature and equivalent resistance .

Case 1a: Low Temperature = 200 K , high equivalent-impedance = 10 ohm and  NF= 3 produces a  BER = (10^-8).

Case 1b: Low Temperature = 400 K , high equivalent-impedance = 10 ohm and  NF= 3 produces a  BER = (10^-4.5).

Case 1c: (Still low impedance) .Low Temperature = 400 K , high equivalent-impedance = 3.3 ohm and  NF= 3 produces a  BER = (10^-3).

Here in the below use cases from Case 2a to 2b  we are trying to capture BER by  varying NOISE FIGURE value.

Case 2a : Temp= 300k , Eqi Res = 33 ohm , NF= 7 DB  produces BER = (10^-8.2)

Case 2b: Temp= 300K , Eqi Res = 33 ohm , NF= 3 DB  produces BER = (10^-15)

3. (90 pts) Laser power and RIN noise

It is obvious that the higher transmitter power, the better is the BER.

Answer:

Here in the below cases from 1a to 1e  we are trying to capture BER by  varying temperature and equivalent resistance and getting BER  below  10^-9 which implies less error in transmission.

Case1a : Temp= 200 , Eqi Res = 10 , NF= 3 DB , Power=-26dbm, RIM=-150db/hz produces BER = (10^-9).

Case1b: Temp= 275 , Eqi Res = 10 , NF= 1 DB , Power=-26dbm, RIM=-150db/hz produces BER = (10^-9.4).

Case1c : Temp= 250 , Eqi Res = 33ohm , NF= 5 DB , Power=-26dbm, RIM=-150db/hz produces a BER = (10^-11)

Case1d:Temp= 275 , Eqi Res = 33 , NF= 4 DB , Power=-26dbm, RIM=-150db/hz produces a BER = (10^-14)

Case1e:Temp= 275 , Eqi Res = 33 , NF= 3 DB , Power=-26dbm, RIM=-150db/hz produces a BER = (10^-15)

Here in the below cases from 2a to 2e   we are trying to capture BER by  varying signal power  and getting BER  increasing that 10^-5 which implies less error in transmission.

We can always find BER as a function of power by using this formula mentioned below,

Plot[Table[Log[10.,BERf[10^(P10)*10^-3,1.5010^br]],{br,8,10,1}],{P,-25,-35},PlotRange→{-15,-3},PlotStyle→{Red,Green,Blue},Frame→True,GridLines→Automatic,AspectRatio→1.5]

Case2a:Temp= 300 , Eqi Res = 100 , NF= 3 DB , Power=-31dbm, RIM=-150db/hz  producing a BER = (10^-5)

Case2b:Temp= 300 , Eqi Res = 100 , NF= 3 DB , Power=-32dbm, RIM=-150db/hz producing a BER = (10^-3.5)

Case2c:Power= -28db, RIN=-150db/hz ,Temp= 300 , Eqi Res = 33 , NF= 5 DB , RIM=-150db/hz producing a BER = (10^-15)

Case2d:Temp= 300 , Eqi Res = 100 , NF= 3 DB , Power=-30dbm, RIM=-150db/hz producing a BER = (10^-8.5)

Case2e:Temp= 300 , Eqi Res = 100 , NF= 3 DB , Power=-29dbm, RIM=-150db/hz producing a BER = (10^-11)

4. (50 pts.) The use of optical pre-amp

Assume that you have a laser power as in 3.1 such that the BER is just worse than 10-5. For example, in the example of 3.1, that power is -31.5 dBm. Show two examples how you can use optical preamp to improve the BER better than 10-9 with different gain and optical bandwidth. Below is both example:

Answer:

Here in the below experiment we are trying to check the BER based on varying the Optical Amplifier and bandwidth. Seems like BER decreases when we use higher amplification .

Case1a: Power= -28db, RIN=-150db/hz ,Temp= 300 , Eqi Res = 33 , NF= 5 DB , RIM=-150db/hz, OPAMP=3,OP BW= 10,producing a BER = (10^-5.2)

Case1b:Power= -28db, RIN=-150db/hz ,Temp= 300 , Eqi Res = 33 , NF= 5 DB , RIM=-150db/hz, OPAMP=7,OP BW= 50 producing a BER = (10^-15)